MS2
Project 1

Andrew Nady

Salma Soliman

### RISC-V-Single-Cycle-Microprocessor

Introduction:

We are designing a RISC-V processor that supports the base integer instruction set according to the specifications found here: https://riscv.org/technical/specifications/

We implemented each instruction according to the specification found in page 130 of the RISC-V Instruction Set Manual –Volume I: Unprivileged ISA and explained in Chapter2 of the same manual.

Design:

- To support JAL/JALR/AUIPC, we added a MUX that’s colored in red in the below pic.

- To support JALR, we added an additional MUX that chooses between the output of the previous mux and the JALR signal.

- We added other singles in the control unit in order to differentiate between some new instructions

  - We added the Aj flag which distinguishes the Auipc and the jal instructions from each other where Aj flag selects from the jal/jalr, Auipc or from the alu

  - We added an i_type flag  where some instructions such as addi can make conflict between sub and addi so we make this flag to differentiate between them.

  - We added lui flag which tells the ALU to shift left  the immediate 12 times.

Algorithm:

In order to support the whole instructions, we did that by dividing them into groups (R-type, I-type,load instruction group, store instruction group, branches group, jal,jalr and AUIPC).

For supporting R- type instructions:

Based on the alu_selection line we choose the output of the ALU to be the result of the operation it does.

case(aluSel) 4'b0000: result = sum; 4'b0001 :result= sub ; 4'b0101 : result= in1 & in2 ; 4'b0100 : result= in1 ¦ in2 ; 4'b0111:result=in1^in2; 4'b1000: result = in1>>in2; //SRL 4'b1010:result = in1>>>in2; //SRA 4'b1001:result = in1<<in2; //SLL 4'b1101:result={(in1<in2)?1:0};//SLT 4'b1111:result={({in1<0?-in1:in1}<{in2<0?-in2:in2})?1:0};//SLTU 4'b0110:result={in2,12'b0}; ///LUI

For supporting I- type instructions:

Based on a flag we created called (I_type flag ). As in the I-type instructions , the processor performs the same operations as R-type instructions and the only difference is the second operand where the ALU takes an immediate instead of a register operand. This flag is only activated in the control unit when the opcode equals the I-type opcode and deactivated for the rest of the instructions. Furthermore, in the ALU control unit we distinguished between the instructions using this flag.

else if (aluop == 2'b10 &&lui_flag==0 && i_type == 0&& instr2 == 0 && intr1 == 3'b000) // R- type instruction (ADD) //instr2 VI aluS = 4'b0000; else if (aluop == 2'b10 &&lui_flag==0 && i_type == 1 && intr1 == 3'b000) // R- type instruction (ADDi) //same aluS as add aluS = 4'b0000;

For supporting B-type instructions:

For supporting the branching, we used the zero flag as an indication that the argument of any branch is true or false. Once it’s true, the zero flag is set to 1, otherwise 0. For the unsigned branches, we settled on the sign first. If the number is negative, we multiply it by -1 and then compare. If it’s positive, we compare directly.

4'b0010 : // branching begin case(func3) 3'b000: if (in1 == in2) zero =1; else zero = 0; // beq zero =1 branch =1 3'b001: if (in1 != in2) zero = 1; else zero = 0; // bne branch =1 zero =0 3'b100: if (in1<in2) zero = 1; else zero = 0; // blt branch=1 3'b101: if (in1>= in2) zero = 1; else zero = 0; //bge branch=1 3'b110: if ({in1<0?-in1:in1}<{in2<0?-in2:in2}) zero = 1; else zero = 0; // bltu branch=1 3'b111: if ({in1<0?-in1:in1}>={in2<0?-in2:in2}) zero = 1; else zero = 0; //bgeu branch=1

For supporting Store-type instructions:

We modified the data memory module by passing function 3 for it so that it could distinguish between instructions. In order to support s-type, we need to activate the memwrite signal coming from the control unit and deactivate memread.

if (MemWrite==1 && func3 ==3'b010) //SW mem[addr] <= data_in; else if (MemWrite==1 && func3 ==3'b001)//sh mem[addr][15:0] <= data_in; else if (MemWrite==1 && func3 ==3'b000)//sb mem[addr][7:0] <= data_in; end

For supporting load-type instructions:

In order to support U-type, we need to activate the memread signal coming from the control unit and deactivate memwrite.

begin if ( func3 == 3'b000) //LB data_out = MemRead ? mem[addr][7:0]: 0; else if ( func3 == 3'b001) //LH data_out = MemRead ? mem[addr][15:0]: 0; else if ( func3 == 3'b010) //LW data_out = MemRead ? mem[addr]:0; else if ( func3 == 3'b100) //LBU data_out = MemRead ?{ 24'h0,mem[addr][7:0]}:0; else if ( func3 == 3'b101)// LHU data_out = MemRead ? {16'h0,mem[addr][15:0]}: 0;

For supporting JAL instruction:

We activated the branch signal in the control unit to allow the pc to branch. Also, we have added a new signal called AJ. it’s a 2 bit signal. The first bit indicates the AUIPC signal and the second bit indicates the J signal related to jal/jalr.

5'b11011: //jal begin branch=1; memread=0; memtoreg=0; aluop=11; memwrite=0; alusrc=1; regwrite=0; i_type=0; lui_flag=0; AJ_control=2'b01;

In the ALU control unit, we added this code that chooses the selection line.

if(aluop==2'b11) //jal aluS=4'b0011;

We activate the zero flag to branch in the ALU module.

4'b0011:zero=1;

For supporting JALR instruction:

5'b11001: //jalr begin branch=0; memread=0; memtoreg=0; aluop=10; memwrite=0; alusrc=1; regwrite=1; lui_flag=0; i_type=0; AJ_control=2'b01;

For supporting AUIPC instruction:

5'b00101: //auipc begin branch=0; memread=0; memtoreg=0; aluop=00; //add as load memwrite=0; alusrc=1; lui_flag=0; regwrite=1; i_type=0; AJ_control=2'b11;

For supporting LUI instruction:

In the ALU module we added this code that loads the register with the upper 20 bits of the immediate and the rest of bits set them to zero.

4'b0110:result={in2,12'b0}; ///LUI

In the control unit we added this code.

5'b01101: //lui begin branch=0; memread=0; memtoreg=0; aluop=10; memwrite=0; alusrc=1; regwrite=1; i_type=0; lui_flag=1; AJ_control=2'b00;

Testing:

We created 4 assembly files in order to test our single cycle processor

R type

1) we created R_typeTest.asm in order to test all the R type instructions,

2) then we converted them to binary to write them inside the instruction memory module

| Assembly | Binary |
| --- | --- |
| lw t0, 0(zero) # 10 ->1010 lw t1, 4(zero) #12 ->1100 lw t2, 8(zero) #1 ->5 add t3,t1,t0 #22 sub t4,t1,t0 #2 and t5,t1,t0 #1000 ->8 xor t6,t1,t0 #0100 ->4 or s0,t1,t0 #1110 ->14 sll s1,t1,t2 #24 slt s2,t1,t0 #0 sltu s3,t1,t0 #0 sra s4,t1,t2 #6 srl s5,t1,t0 #6 | 00000000000000000010001010000011 00000000010000000010001100000011 00000000100000000010001110000011 00000000010100110000111000110011 01000000010100110000111010110011 00000000010100110111111100110011 00000000010100110100111110110011 00000000010100110110010000110011 00000000011100110001010010110011 00000000010100110010100100110011 00000000010100110011100110110011 01000000011100110101101000110011 00000000010100110101101010110011 |

Screenshot

register

I type

| Assembly | Binary |
| --- | --- |
| lw t1, 4(zero) #12 ->1100 addi t3,t1,10 #22 andi t5,t1,10 #1000 ->8 xori t6,t1,10 #0100 ->4 ori s0,t1,10 #1110 ->14 slli s1,t1,2 #24 slti s2,t1,10 #0 sltiu s3,t1,10 #0 srai s4,t1,2 #6 srli s5,t1,10 #6 | 00000000010000000010001100000011 00000000101000110000111000010011 00000000101000110111111100010011 00000000101000110100111110010011 00000000101000110110010000010011 00000000001000110001010010010011 00000000101000110010100100010011 00000000101000110011100110010011 01000000001000110101101000010011 00000000101000110101101010010011 |

Screenshot

Registers

U type&jalr&jalr

| Assembly | Binary |
| --- | --- |
| lw t1,0(zero)#0 lw t2,4,(zero) #12 lw t3,8(zero) #10 jal jumb addi t1,t1,1 jumb: auipc t2,10000 lui t3,10000 addi t4,zero,0 jalr t0,t4,36 addi t1,t1,1 jumb2: sw t3,12(zero) #10 pc=36 | 00000000000000000010001100000011 00000000010000000010001110000011 00000000100000000010111000000011 00000000100000000000000011101111 00000000000100110000001100010011 00000010011100010000001110010111 00000010011100010000111000110111 00000000000000000000111010010011 00000010010011101000001011100111 00000000000100110000001100010011 00000001110000000010011000100011 |

Screenshot

Registers and PC

S type

| Assembly | Binary |
| --- | --- |
| lw t1,0(zero)#0 lw t2,4,(zero) #12 lw t3,8(zero) #10 sw t1,12(zero)#0 sh t2,16,(zero) #12 sb t3,18(zero) #10 | 00000000000000000010001100000011 00000000010000000010001110000011 00000000100000000010111000000011 00000000011000000010011000100011 00000000011100000001100000100011 00000001110000000000100100100011 |

Screenshot

Memory: